60 HMT Pocket Acoustics

Fitting for top-running wooden doors up to 60 kg (132 lbs.), with surface mounted running

track or running track flush with the ceiling. Sound attenuation. Ceiling mounting. Wall

pocket solution.

Product-Highlights

Comfort

Outstanding living comfort thanks to effective exclusion of sound,

drafts, odors and unwanted light incidence

Flexibility

Ceiling-integrated or surface-mounted pocket solutions possible

Productivity Door can be installed in pocket after construction is complete

Technical guidelines

Max. Door weight

60 kg (132 lbs.)

Door thickness

Max. Door height

Door width

Opening width (LMB)

Height adjustable

Soft close

39–45 mm (1 17/32'' to 1 25/32'')

2500 mm (8' 2 7/16'')

800–1300 mm (2' 7 1/2'' to 4' 3 3/16'')

750–1250 mm (2' 5 17/32'' to 4' 1 7/32'')

+/- 2 mm (3/32'')

No

Door material

Wood

Finish color of visible profiles

Aluminum anodized

System options

Application area

Ceiling mounting

Ceiling mounting recessed

Top-running

Yes

Yes

Yes

Utilization light / semi-public access

Utilization medium / semi-public access

Utilization high / public access

Yes

Yes

Yes

Tests

Building hardware – Hardware for sliding doors and folding doors according to EN 1527 / 2020

– Duration of functionality: Class 6 (highest class = 100,000 cycles)

Warranty

With the exception of parts subject to wear and tear, Hawa warrants the flawless functioning

of the products delivered by it, as well as the durability of all parts, for a period of 2 years

commencing from the transfer of risk.

Product Design

Hawa Porta 60 HMT Acoustics consisting of running track, running gear with friction bearing

rollers, SoftStop, stopper with retention spring, running gear support, horizontal seal set,

vertical seal.

Optional:

(….) Hawa Porta 60/100 assembly set

(….) Spring loaded buffer

Hawa Sliding Solutions AG

Untere Fischbachstrasse 4, 8932 Mettmenstetten, Switzerland

Phone +41 44 787 17 17, info@hawa.com, www.hawa.com

1

60 HMT Pocket Acoustics

Interfaces

Door leaf

–

–

–

–

Suspension profile and sealing groove, top (H × W) 30 × 27 mm (1 3/16'' × 1 1/16'')

Guide and sealing groove, bottom (H × W) 30 × 23 mm (1 3/16'' × 29/32'')

Screw fixing of the suspension profiles and horizontal seals

Groove for seal, vertical 4 x 4 mm (5/32'' × 5/32'')

Vertical seal level

–

Leading surface for vertical seal must be provided by the customer

(min. 6 mm (1/4'') distance from wall structure)

Gap size to floor

– Air gaps of 8–12 mm (5/16'' to 15/32'') can be taken up by the seal

Pocket construction

– Pocket construction must be taken care of by the customer

Hawa Sliding Solutions AG

Untere Fischbachstrasse 4, 8932 Mettmenstetten, Switzerland

Phone +41 44 787 17 17, info@hawa.com, www.hawa.com

2